1.传统几何方法

传统SLAM方法主要依据传感器观测构建显式约束，并通过非线性优化估计运动状态与环境结构。Mur-Artal等人提出的ORB-SLAM在跟踪、局部建图、重定位和回环检测中统一使用ORB特征，通过关键帧管理、局部束调整与回环优化实现实时单目定位建图，为后续特征法系统建立了重要的整体框架。[1]在此基础上，ORB-SLAM2扩展了双目和RGB-D输入，利用视差或深度观测获得度量尺度，并通过统一的几何优化后端处理不同相机配置。[2]ORB-SLAM3进一步融合视觉与惯性测量，引入多地图创建、识别和合并机制，使系统能够在跟踪中断后继续建图，并在重新访问已知区域时利用历史观测改善轨迹一致性。[3]

除基于特征匹配的路线外，Engel等人提出的DSO直接利用图像亮度信息，在滑动窗口内联合优化相机位姿、点逆深度及光度参数。它无需依赖特征描述子，能够利用具有有效梯度的图像区域，但对光度模型和场景静态性有较强要求。原始DSO主要是局部运动与结构估计方法，属于视觉里程计。[4]为缓解单目视觉的尺度不确定性并提高快速运动时的状态估计稳定性，Qin等人提出VINS-Mono，将视觉特征重投影约束与IMU预积分约束纳入紧耦合滑动窗口优化，并通过回环检测和位姿图优化降低长期漂移。由于运行时需要相机与IMU，VINS-Mono属于视觉惯性系统。[5]

在稠密建图方面，Yang等人提出FD-SLAM，将RGB-D帧到模型的稠密配准与基于ORB特征的跨子图关联结合：前者用于局部运动估计和TSDF子图构建，后者为全局地图优化提供长期约束。系统通过束调整联合优化特征点与子图位姿，以缓解稠密建图中的累计漂移。其回环检测和重定位模块还使用学习得到的三维特征，因此该方法的定位建图主体由显式几何约束驱动，但并非完全不含学习模块。[6]

上述工作共同表明，特征重投影、光度一致性、惯性测量和稠密配准能够为状态估计提供互补约束；回环检测与全局优化则是控制长期漂移的重要环节。同时，这些方法的输入模态和任务边界并不相同：DSO主要解决视觉里程计问题，VINS-Mono使用视觉与惯性输入，FD-SLAM侧重RGB-D稠密重建。因此，对其性能的讨论应结合相应传感器配置、地图形式和评价任务展开。

2.学习式定位与几何优化结合

[1] ORB-SLAM: a Versatile and Accurate Monocular SLAM System

[2] ORB-SLAM2: an Open-Source SLAM System for Monocular, Stereo and RGB-D Cameras

[3] ORB-SLAM3: An Accurate Open-Source Library for Visual, Visual–Inertial, and Multimap SLAM

[4] VINS-Mono: A Robust and Versatile Monocular Visual-Inertial State Estimator

[5] VINS-Mono: A Robust and Versatile Monocular Visual-Inertial State Estimator

[6] FD-SLAM: 3-D Reconstruction Using Features and Dense Matching